# Numerical Method

## Finite-volume framework

We solve the two-dimensional compressible Euler equations on an unstructured triangular mesh with a cell-centred finite-volume method. For each cell Ωi the semi-discrete update reads

dUidt=−1Ωif∈∂Ωi​F​WfL, WfR; nf f,

where Ui is the cell-averaged conservative state, W=ρ,u,v,pT the primitive state, nf the outward face normal, and f the face length. The numerical flux F is a fixed rotated-Roe hybrid and is not modified in this work; the contribution is confined to the reconstruction of the face states WfL,WfR. Time integration uses a strong-stability-preserving Runge–Kutta scheme (SSP-RK2/RK3).

## Linear least-squares gradient

A single first-order (P1) gradient is computed per cell by weighted least squares over the face/vertex neighbour set Ni,

∇Wi=k∈Ni​wik dikdikT−1k∈Ni​wik dik Wk−Wi,  dik=xk−xi, wik=dik−p.

No second derivatives (Hessian) are used, so the per-step cost matches a standard linear scheme. Mesh non-orthogonality and skewness are accounted for geometrically through the true centroid offsets dik and the vertex offsets xv−xi used in the limiter below.

The unlimited face value is

WfL=WL+ψL ∇WL​⋅xf−xL,  ψ∈0,ψmax,  ψmax=2.

## Vertex local-maximum-principle limiter

Monotonicity is enforced by a vertex-based local-maximum-principle (LMP) limiter. For every vertex v of cell i the reconstructed value is required to lie within the min/max of the cells sharing that vertex, Wvmin≤Wi+ψ ∇Wi​⋅xv−xi≤Wvmax. Solving for the largest admissible ψ gives

ψLMP=minvmax​0, rvsv,  sv=∇Wi​⋅xv−xi, rv=Wvmax−Wi,sv≥0,Wi−Wvmin,sv<0,

capped at ψmax=2. Because the reconstructed value never exceeds the neighbouring extrema, no new extrema are created and the scheme is monotone even when ψ approaches the compressive limit 2. This LMP wrapper is the core of the T-MLP-u construction.

Three admissibility regimes are distinguished and used throughout:

ψ≤ψLMP⏟(I) bounded compression | ψLMP<ψ≤1⏟(II) smooth-extremum un-clip | ψ>ψLMP⏟(III) anti-diffusion: excluded.

Regimes (I) and (II) are admissible; regime (III), reconstruction beyond the bound (injected anti-diffusion), is never used.

## Feature detection

Smoothness gate. A cell is flagged as a genuine discontinuity only when both the normalized least-squares residual and the stencil bimodality gap exceed a threshold θ,

Ri=k​​(Δϕk−∇ϕi​⋅dik)2k​Δϕk2, Gi=maxkϕk+1−ϕkϕm−ϕ1, sharp⇔Ri≥θ ∧ Gi≥θ,

with Δϕk=ϕk−ϕi and ϕ1≤…≤ϕm the sorted stencil values. The residual measures non-planarity, the gap measures bimodality; together they separate a true step from smooth curvature.

Physical gates. Using the dilatation ϑ=∇​⋅u and vorticity ω=∂xv−∂yu, a compression-dominance indicator and a relative pressure jump are formed,

D=max0,−ϑ2max0,−ϑ2+ω2+ε,  P=maxkpk−pipk+pi,

from which a shock gate and a non-shock rotation gate are defined with a smoothstep blend S,

gs=SD SP,  cω=1−D1−gs.

The pressure-jump factor is essential: a slip line is a contact (P​≈​0) whereas a shock has large P, so gs distinguishes a rolling shear layer from a true shock and prevents vortices from being misclassified.

## Component-wise limiter

With the far-upwind ratio r=Δ+/Δ−, Δ−=∇WL​⋅xR−xL, Δ+=WR−WL, two base limiters are used: the smooth TVD limiter ψvLr=2r/1+r and a compressive STACS-type limiter ψMr=min​ψSBr, 2r/Co, where ψSB is the superbee limiter and Co a compression parameter. The target limiter is assigned per variable,

ψt=1−gc ψvL+gc ψM,ρ,ψvL,p,(shear/vortex treatment below),u,v,

where gc is the contact gate. A shock cap ψ≤1−κ gs keeps all variables conservative across shocks, suppressing carbuncle and post-shock oscillations.

## Shear-layer compression and vortex-core un-clipping

For the velocity components the limiter exploits the admissible compressive range while remaining monotone. The shear-layer compression blends the low-dissipation smooth value toward the compressive limiter in proportion to the rotation gate,

ψt vel=1−g⋅1+g ψM,  g=β cω,  ψ=min​ψt vel, ψLMP,

so that ψ≤ψLMP (regime I): the shear sheet is sharpened toward its physical thin-vortex-sheet limit without creating new extrema.

Because a developing Kelvin–Helmholtz core is a smooth local extremum, the LMP bound drives ψLMP​→​0 there and clips the core. A smooth-extremum un-clipping (MLP-u2 type), restricted to velocity and gated by cω, relaxes the bound,

ψLMP relax=min​2, ψLMP1+α cω,  ψ=min​ψt vel, ψLMP relax.

Density and pressure retain the strict bound ψLMP, which preserves positivity of the fixed flux. The fixed-fraction relaxation may be replaced by a Venkatakrishnan-type dimensionless margin ε2=K Δx3 to make the un-clipping resolution-consistent.

## Positivity and time integration

After reconstruction, density and pressure face values are floored, ρf≥η ρi and pf≥η pi, and the update is advanced with an SSP Runge–Kutta scheme. A Zhang–Shu positivity-preserving pullback can be appended as a final admissibility layer.